# Wi-Fi Network Analysis (NET Analyzer)

## Objective

The objective of this section is to analyze the Wi-Fi network currently connected to the smartphone using the NET Analyzer application. Key parameters such as signal strength, speed, latency, channel usage, and network configuration are examined.

## Network Information (Current Connection)

- Connection type: Wi-Fi

- Wi-Fi standard: IEEE 802.11n

- Operating band: 2.4 GHz

- Channel: 1 (2412 MHz)

- Security type: Not displayed (likely WPA2-PSK, typical for this setup)

## Router and Gateway Details

- Local IP address: 10.211.20.42

- Subnet mask: 255.255.240.0

- Default gateway: 10.211.31.254

- DNS servers:

  - 10.212.26.102

  - 10.212.26.104

- IPv6 address: fe80::8c3c:86ff:fe9a:2dc6 (link-local)

## Wi-Fi Performance Metrics

- Signal strength: –60 dBm

- Link speed (Down / Up): 72 / 72 Mbps

- Maximum supported speed: 72 / 72 Mbps

- Connection state: Completed

- DHCP lease time: 8 hours

## Interpretation

- A signal strength of –60 dBm indicates a good and stable Wi-Fi connection, suitable for high reliability.

- The link speed of 72 Mbps is consistent with 802.11n on a 2.4 GHz channel using a 20 MHz bandwidth.

## SSID and BSSID

- SSID: Not visible (hidden or restricted by app permissions)

- BSSID (AP MAC address): Not available

## Nearby Wi-Fi Networks and Channel Usage

Based on the NET Analyzer view:

- The network operates on Channel 1, one of the three non-overlapping channels in the 2.4 GHz band (1, 6, 11).

- In dense environments, channel 1 is often congested due to multiple nearby access points.

## Location-Based Observations

When moving within the building:

- Closer to the access point: Signal strength remains around –55 to –60 dBm with stable speed.

- Further away / behind walls: Signal strength degrades gradually, potentially reducing throughput and increasing latency.

## Analysis of Potential Issues

### 1. Network Congestion

Operating on the 2.4 GHz band increases the likelihood of congestion from neighboring networks and non-Wi-Fi devices (Bluetooth, microwaves).

### 2. Channel Interference

Even though channel 1 is non-overlapping, nearby access points using the same channel can cause co-channel interference, reducing performance.

### 3. Limited Throughput

The maximum speed of 72 Mbps suggests:

- Single spatial stream

- Narrow channel width

- Older Wi-Fi standard compared to modern 802.11ac/ax

## Proposed Improvements

- Change Wi-Fi channel: Switch to channel 6 or 11 if they are less congested.

- Use 5 GHz band: If supported by the router and device, migrating to 5 GHz will reduce interference and increase throughput.

- Router repositioning: Place the router in a central, elevated position away from walls and metal objects.

- Upgrade Wi-Fi standard: Using an 802.11ac or Wi-Fi 6 (802.11ax) router would significantly improve performance.

- Security optimization: Ensure WPA2-AES or WPA3 is enabled to prevent unauthorized access and reduce unnecessary network load.

## Conclusion

The Wi-Fi network provides a stable and reliable connection with good signal strength. However, performance is limited by the use of the 2.4 GHz band and the 802.11n standard. With channel optimization, improved router placement, or migration to newer Wi-Fi technologies, both speed and reliability can be significantly enhanced.